Update Instructions – QTL1461

- Install the Quarch USB drivers if not already done:

http://quarch.com/downloads?field_file_type_tid=113

- Download and install the updater utility:

http://quarch.com/file/torridon-updater

- Connect module to the PC via USB

- Power up the module

  - USB may take a while to identify, if windows tries to search online for the drivers, you can skip this.

- Run the updater program

Update FPGA:

- The module will appear in the left list at “QTL1461”.  Select it and click ‘FPGA’

  - Navigate to the FPGA update file for this module and select it

  - Click ‘Upload’.  This will take a few minutes to complete

Update the LAN Firmware:

IF you are using v4.002 firmware or higher, the LAN processor USB is turned off by default, this avoids confusing customers with multiple devices appearing.  To update, you must turn it on. This is done with the “conf:ethernet:term on” command from the main terminal.  You only have to do this if you do NOT see the QTL1537 item in the left menu selection.

- Select the “QTL1537” module in the left list and click the ‘>>’ button.  This will start a new USB device, when this has identified in windows, an entry for the module will appear in the right hand list.

  - Navigate to the LAN Firmware update file for this module and select it

  - Click ‘Upload’.  This will take a few seconds to complete

Update the Main Firmware:

- Select the “QTL1461” module in the left list and click the ‘>>’ button.  This will start a new USB device, when this has identified in windows, an entry for the module will appear in the right hand list.

  - Navigate to the Main Firmware update file for this module and select it

  - Click ‘Upload’.  This will take a few seconds to complete

- When complete, you must power cycle then run the “conf:default factory” command from the main terminal.  This will ensure all the new LAN settings are updated to the EEPROM memory.