Update Instructions – QTL1658

- Install the Quarch USB drivers if not already done:

  - http://quarch.com/system/files/asset_files/Torridon_USB_Driver_Win_XP_Win_7.zip

- Download and install the updater utility:

  - http://quarch.com/system/files/asset_files/Torridon%20Updater%202%20v2.5.zip

- Connect module to the PC via USB, using an Interface Kit or Array Controller

- Power up the module

  - USB may take a while to identify, if windows tries to search online for the drivers, you can skip this.

- Run the updater program

Update FPGA:

- The module will appear in the left list at “QTL1658”.  Select it and click ‘FPGA’

  - Navigate to the FPGA update file for this module and select it

  - Click ‘Upload’.  This will take a few minutes to complete

NOTE: If you have multiple QTL1658 modules connected at the same time (Perhaps via a 4-Port Controller), you can select all the modules at the same time before clicking the ‘Update’ button

Update Firmware:

- Select the “QTL16958” module in the left list and click the ‘>>’ button.  This will start a new USB device, when this has identified in windows, an entry for the module will appear in the right hand list.

  - Navigate to the Main Firmware update file for this module and select it

  - Click ‘Upload’.  This will take a few seconds to complete