Update Instructions – QTL1824 (-03 and up ONLY)

Note that -02 modules cannot be updated with this pack.  Upgrade from -02 to -03 is possible.  Please contact Support@Quarch.com for details

- Install the Quarch USB drivers if not already done:

  - http://quarch.com/file/torridon-driver-win7

  - http://quarch.com/file/torridon-driver-win8

- Download and install the updater utility:

  - http://quarch.com/file/torridon-updater

- Connect module to the PC via USB, using an Interface Kit or Array Controller

- Power up the module

  - USB may take a while to identify, if windows tries to search online for the drivers, you can skip this.

- Run the updater program

Update FPGA:

- The module will appear in the left list at “QTL1824”.  Select it and click ‘FPGA’

  - Navigate to the FPGA update file for this module and select it

  - Click ‘Upload’.  This will take a few minutes to complete

NOTE: If you have multiple QTL1824 modules connected at the same time (Perhaps via a 4-Port Controller), you can select all the modules at the same time before clicking the ‘Update’ button

Update Firmware:

- Select the “QTL1824” module in the left list and click the ‘>>’ button.  This will start a new USB device, when this has identified in windows, an entry for the module will appear in the right hand list.

  - Navigate to the Main Firmware update file for this module and select it

  - Click ‘Upload’.  This will take a few seconds to complete